BLIND MAN SMART STICK

Arduino Nano  |  Ultrasonic Sensor  |  Morse Code Buzzer Alert

Assistive Technology Project Documentation

# 1. Experiment

The Blind Man Smart Stick is an assistive technology project built using an Arduino Nano microcontroller, an HC-SR04 ultrasonic distance sensor, and an active buzzer. The system is designed to help visually impaired individuals navigate their surroundings safely.

When the ultrasonic sensor detects an obstacle within a predefined distance (100 cm by default), the buzzer plays the Morse code sequence for 'ALRT' — acting as a distinctive audio alert that informs the user of a nearby obstacle. The pattern repeats as long as the obstacle remains in range.

# 2. Requirements

## Software Requirements

- Arduino IDE (version 1.8.x or 2.x) — free download from arduino.cc

- USB driver for CH340G chip (if using clone Arduino Nano)

- Serial Monitor (built into Arduino IDE) for debugging

## Hardware Requirements

- 5V regulated power (USB or 9V battery via VIN pin)

- Stable breadboard connections or soldered circuit

- A walking stick, cane, or PVC pipe to mount components

- Electrical tape or zip ties for secure mounting

## Skills Required

- Basic Arduino programming knowledge

- Understanding of digital I/O pins

- Basic circuit wiring on breadboard

# 3. Components

The following components are required to build the Blind Man Smart Stick:

| # | Component | Quantity |
| --- | --- | --- |
| 1 | Arduino Nano | 1 |
| 2 | HC-SR04 Ultrasonic Sensor | 1 |
| 3 | Active Buzzer | 1 |
| 4 | Breadboard | 1 |
| 5 | Jumper Wires | 10+ |
| 6 | USB Cable | 1 |
| 7 | Walking Stick / PVC Pipe | 1 |
| 8 | 9V Battery | 1 |
| 9 | Electrical Tape | 1 roll |

Note: An Arduino Uno or Arduino Mega can be used as an alternative to the Nano, but the Nano is preferred for its compact size.

# 4. Connectivity (Pin Diagram)

Connect the components to the Arduino Nano as per the table below. Ensure all GND connections share a common ground on the breadboard.

| Component | Component Pin | Arduino Nano Pin |
| --- | --- | --- |
| HC-SR04 Ultrasonic | VCC | 5V |
| HC-SR04 Ultrasonic | GND | GND |
| HC-SR04 Ultrasonic | TRIG | D9 |
| HC-SR04 Ultrasonic | ECHO | D10 |
| Active Buzzer | Positive (+) | D8 |
| Active Buzzer | Negative (–) | GND |
| 9V Battery | Positive (+) | VIN |
| 9V Battery | Negative (–) | GND |

# 5. Steps Procedure

## Step 1 — Install Arduino IDE

- Download Arduino IDE from https://www.arduino.cc/en/software

- Install the IDE and open it

- Connect the Arduino Nano via USB cable to your computer

- Go to Tools > Board > Arduino Nano

- Select the correct COM Port under Tools > Port

## Step 2 — Assemble the Circuit

- Place the Arduino Nano on the breadboard

- Connect the HC-SR04 VCC to Arduino 5V and GND to Arduino GND

- Connect HC-SR04 TRIG to Arduino D9 and ECHO to Arduino D10

- Connect the buzzer positive pin to Arduino D8 and negative to GND

- If using a 9V battery, connect the battery clip positive to VIN and negative to GND

- Double-check all connections before powering on

## Step 3 — Upload the Code

- Open Arduino IDE and create a new sketch

- Copy the full code from Section 6 of this document

- Paste it into the IDE editor

- Click the Verify button (checkmark icon) to compile

- Click the Upload button (arrow icon) to flash to the board

- Wait for the 'Done uploading' message

## Step 4 — Test the System

- Open Serial Monitor (Ctrl + Shift + M) to view distance readings

- Set baud rate to 9600 in Serial Monitor

- Place your hand in front of the sensor at various distances

- When your hand is within 100 cm, the buzzer should play the ALRT Morse code

- Adjust thresholdDistance variable in the code to change sensitivity

## Step 5 — Mount on Stick

- Securely tape the breadboard and components to the top of the walking stick

- Position the ultrasonic sensor at the front of the stick, facing forward

- Attach the buzzer near the handle area for easy hearing

- Route the wires neatly using electrical tape or zip ties

- Connect the 9V battery for portable operation

## Morse Code Alert Pattern

When an obstacle is detected, the buzzer plays the following Morse code pattern for 'ALRT':

| Letter | Morse Code | Buzzer Pattern |
| --- | --- | --- |
| A | · — | Short beep + Long beep |
| L | · — · · | Short + Long + Short + Short |
| R | · — · | Short + Long + Short |
| T | — | Long beep only |

Timing: Dot = 100 ms | Dash = 300 ms | Letter gap = 300 ms | Word gap = 700 ms

# 6. Coding

Below is the complete Arduino sketch for the Blind Man Smart Stick. The code is well-commented for easy understanding and modification.

blind_man_stick.ino

// Blind Man Stick - Arduino Nano + Ultrasonic Sensor + Buzzer

// Plays Morse code 'ALRT' when obstacle is within threshold distance

const int trigPin = 9;             // Ultrasonic Sensor Trigger Pin

const int echoPin = 10;            // Ultrasonic Sensor Echo Pin

const int buzzerPin = 8;           // Buzzer Pin

const int thresholdDistance = 100; // Alert if obstacle within 100 cm

const int dotDuration = 100;       // Morse dot = 100 ms

const int buzzerFreq = 1000;       // Buzzer tone frequency in Hz

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(buzzerPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

long duration, distance;

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = (duration * 0.0343) / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

if (distance > 0 && distance <= thresholdDistance) {

playALRT();

delay(dotDuration * 7);

}

delay(100);

}

void playDot() {

tone(buzzerPin, buzzerFreq);

delay(dotDuration);

noTone(buzzerPin);

delay(dotDuration);

}

void playDash() {

tone(buzzerPin, buzzerFreq);

delay(dotDuration * 3);

noTone(buzzerPin);

delay(dotDuration);

}

void letterSpace() { delay(dotDuration * 2); }

void playA() { playDot(); playDash(); letterSpace(); }

void playL() { playDot(); playDash(); playDot(); playDot(); letterSpace(); }

void playR() { playDot(); playDash(); playDot(); letterSpace(); }

void playT() { playDash(); letterSpace(); }

void playALRT() { playA(); playL(); playR(); playT(); }

## Code Explanation

- trigPin (D9): Sends a 10 µs ultrasonic pulse to the HC-SR04

- echoPin (D10): Listens for the reflected pulse and measures duration

- distance = (duration × 0.0343) / 2: Converts time to centimeters using speed of sound

- thresholdDistance = 100: Alert triggers when obstacle is within 100 cm

- playALRT(): Calls individual letter functions A → L → R → T in sequence

- dotDuration = 100 ms: Base unit for all Morse timing calculations

- buzzerFreq = 1000 Hz: Frequency of the buzzer tone

# 7. Buy Links — India

The following Indian electronics websites stock all required components. Most offer pan-India shipping with delivery in 2–5 business days.

| Component | Search Term / URL | Price |  |
| --- | --- | --- | --- |
| Arduino Nano | https://robocraze.com/products/nano-development-board-compatible-with-arduino | 198 |  |
| HC-SR04 Sensor | https://robocraze.com/products/hc-sr-04-ultrasonic-sensor | 76 |  |
| Active Buzzer 5V | https://robu.in/product/5v-passive-buzzer-5pcs/#tab-robu_other_tab | 38 |  |
| Jumper Wires | https://robocraze.com/products/jumper-wire-set-m2m-m2f-f2f-40-pcs-each | 128 |  |
| Breadboard(optional) | https://robocraze.com/products/half-breadboard | 35 |  |
| Battery case | https://robocraze.com/products/18650-2-cell-holder | 21 |  |
| 18650 batteries - 2 | https://robocraze.com/products/3-7v-1200mah-18650-battery | 40 |  |
| On/off switch | https://robocraze.com/products/2-pin-mini-on-off-spst-rocker-switch-19-x-13mm | 25 |  |
| USB cable | https://robocraze.com/products/usb-a-b-cable-3-0-cm | 33 |  |
|  | total | 594˜700 |  |

Circuit Diagram